HERBERT & SONS LIMITED,       Angel Road Works, Edmonton, London, N.18.

February, 1968.

LION ELECTRONIC CD WEIGHER

The basic unit comprises a weigher of sensitive but conventional design operating a patented device and an encoding unit.  This device converts weight readings into electrical pulses which drive an electronic computing unit controlling a numerical indicator panel.

DESCRIPTION:

The weighing and electronic equipment are built into a work counter 5’x2’3”x2’10” with control panel, weight plate and numerical indicator conveniently positioned on the working surface.

NUMERICAL INDICATION PANEL:

This panel positioned conveniently above the counter surface for easy reading indicates the weight, price per pound and computed price in neon lit figures ¾” high in three separate windows.  Additionally, a further panel can be positioned remotely from the weighing unit such as in a supervisor’s office or check-out counter.

Control Panel

Price selection is operated by two simple rotary switches, one for shillings one for pence.  Two separate button switches are provided one to activate a tare device and the other to control the printout should this be required.

Price Range

Price indication can be provided by 1d  to 39/11 per lb. in the present currency or by New Half Pennies to 199 pence per lb. in decimal currency.  The conversion to decimal currency is by simple replacement of “plug-in” decades and can be carried out in a few minutes.

Capacity

10 lbs. x ¼ oz. divisions.

Weight Plate

Size 18” x 11”.  This is positioned only 2½” above the working surface.  The lifting of goods is thus virtually eliminated.

Computing Unit

The most recently developed type of computer is used incorporating miniature solid state electronic techniques.

Speed of Weighing

If used for normal retail selling the scale would operate at the same speed as any conventional weighing machine.  If used in a pre-packing line, however, because the computer incorporates a built-in memory at least 12 weighings per minute could be maintained.

Printed Record

Whereas the machine can be used as a conventional weigher it can also be provided with a feedout to operate a printer.  This will issue pre-printed tickets from a roll showing price per pound, weight and computer price.